Multiplexed cryo-CMOS control of an isolated double quantum dot

Mathieu Darnas1, Mathilde Ouvrier-Buffet1,2, Antoine Faurie2, Brian Martinez2, Jean-Baptiste Casanova3, Benoît Bertrand2, Candice Thomas2, Jean Charbonnier2, Jean-Philippe Michel2, Bruna Cardoso Paz4, Yvain Thonnart3, Franck Badets2, Franck Balestro1, Matias Urdampilleta1, Tristan Meunier4 and Baptiste Jadot2

1Univ. Grenoble Alpes, CNRS, Grenoble INP, Institut Néel, 38042 Grenoble, France

2Univ. Grenoble Alpes, CEA, Leti, F-38000 Grenoble, France

3Univ. Grenoble Alpes, CEA, List, F-38000 Grenoble, France

4Quobly, Grenoble, France

Scalable spin-based quantum computing demands precise and stable control of a large number of gate-defined quantum dots while minimizing wiring complexity and thermal load [1]. Control architectures based on sample-and-hold (SH) multiplexing techniques offer a promising solution by enabling sequential programming of several gate voltages using a limited number of input lines [2, 3, 4]. However, the compatibility of such dynamic voltage refreshing with the stringent stability, noise, and speed requirements of quantum dot operation is an active subject of study.

We experimentally demonstrate that a multiplexing cryo-CMOS circuit can reliably bias a silicon double quantum dot (DQD) at 0.5K. Exploiting the isolated regime, we show deterministic loading and isolation of four electrons and stable access to all five charge configurations from (4,0) to (0,4), despite the sequential voltage refreshing. We further demonstrate rapid voltage pulsing across an inter-dot transition, resolving single-electron tunneling events and stochastic switching at the (1,3) – (0,4) transition. These results confirm that SH-based multiplexed control is compatible with both static biasing and pulsing of isolated quantum dots, representing an important milestone toward scalable cryogenic control architectures for large-scale spin-qubit processors.

[1] A. Chatterjee, P. Stevenson, S. De Franceschi, A. Morello, N. P. de Leon, and F. Kuemmeth, Semiconductor qubits in practice, Nature Reviews Physics 3, 157 (2021).

[2] S. K. Bartee, W. Gilbert, K. Zuo, K. Das, T. Tanttu, C. H. Yang, N. Dumoulin Stuyck, S. J. Pauka, R. Y. Su, W. H. Lim, S. Serrano, C. C. Escott, F. E. Hudson, K. M. Itoh, A. Laucht, A. S. Dzurak, and D. J. Reilly, Spin-qubit control with a milli-kelvin CMOS chip, Nature 643, 382 (2025)

[3] L. Enthoven, J. van Staveren, J. Gong, M. Babaie, and F. Sebastiano, A 3V 15b 157μW Cryo-CMOS DAC for Multiplexed Spin-Qubit Biasing, in 2022 IEEE Symposium on VLSI Technology and Circuits (VLSI Technology and Circuits) (2022) pp. 228–229.

[4] S. Subramanian, T. M. Mladenov, S. Schaal, B. Patra, L. Lampert, N. K. Robinson, J. Roberts, and S. Pellerano, A Scalable mK Cryo-CMOS Demultiplexer Chip for Voltage Biasing and High-Speed Control of Silicon Qubit Gates, in 2024 IEEE Symposium on VLSI Technology and Circuits (VLSI Technology and Circuits) (2024) pp. 1–2.