# Enhanced Coherence of an Industry-Compatible Silicon Spin Qubit through Correlated Noise Suppression

Authors: Petar Tomić¹, Patrick Bütler¹, Yuze Wu¹, Bart Raes², Clement Godfrin², Stefan Kubicek², Julien Jussot², Yann Canvel², Yannick Hermans², Yosuke Shimura², Roger Loo², Sofie Beyne², Gulzat Jaliel², Thomas Van Caekenberghe²,³, Vukan Levajac², Danny Wan², Kristiaan De Greve²,⁴, Wister Wei Huang¹,⁵, Klaus Ensslin¹, and Thomas Ihn¹

Affiliations:

¹ Laboratory for Solid State Physics, ETH Zürich, Switzerland.
² IMEC, Leuven, Belgium.
³ KU Leuven, Department of Electrical Engineering (ESAT), Belgium.
⁴ Proximus Chair in Quantum Science and Technology, KU Leuven, Belgium.
⁵ National University of Singapore, Singapore.

### Abstract

Silicon spin qubits are a leading candidate for large-scale quantum computing due to their compatibility with industrial CMOS manufacturing processes. While highly optimized industrial environments can successfully minimize material defects, the specific limits of quantum coherence in these environments—and how to overcome them—remain critical areas of exploration. In this work, we demonstrate that a combination of a low-noise, 300 mm-compatible spin-qubit process flow and the strategic selection of qubit encoding can significantly enhance coherence.

We report on the operation of a singlet-triplet (S-T0) qubit fabricated on a state-of-the-art 300 mm CMOS platform featuring polysilicon gates. Our results show that the devices benefit from an exceptionally quiet electrical environment, with measured detuning noise amplitudes as low as δεrms=2.2μeV. Through noise spectroscopy and cross-correlation analysis between neighboring dots, we identify strong zero-phase correlations in the charge noise. We demonstrate that the S-T0 basis effectively rejects these common-mode fluctuations, leading to a substantial suppression of dephasing.

By leveraging this correlated noise environment, we achieve a Hahn-echo coherence time of T2Hahn=4ms. To the best of our knowledge, this is the longest T2Hahn reported to date for any gate-defined quantum dot spin qubit. These findings highlight that tailoring qubit encoding to the specific spatial correlations of the noise landscape is a powerful strategy for scaling foundry-fabricated quantum processors toward practical error-correction thresholds.